SECTION 02285

SOIL TREATMENT FOR SUBTERRANEAN TERMITE CONTROL

# 1	GENERAL

## 1.1	SUMMARY (Not Applicable)

## 1.2	REFERENCES (Not Applicable)

## 1.3	SUBMITTALS

Government approval is required for submittals with a "GA" designation; submittals having an "FIO" designation are for information only.  The following shall be submitted in accordance with Section 01300 SUBMITTAL DESCRIPTIONS:

SD-01 Data

Qualifications; [_____].

Qualifications of the pesticide applicator.

SD-06 Instructions

Pesticides; [_____].

Manufacturer's label and Material Safety Data Sheet (MSDS) for pesticides proposed for use.

SD-14 Samples

Pesticides; [_____].

Samples of the pesticides used in this work, upon request.

## 1.4	QUALIFICATIONS OF PESTICIDE APPLICATORS

The pesticide applicator's principal business shall be pest control and the pesticide applicator shall be State certified in the U.S. Environmental Protection Agency (EPA) pesticide applicator category which includes structural pest control, and certified in the State.

## 1.5	DELIVERY, STORAGE, AND HANDLING

Pesticides shall be delivered to the project site in sealed and labeled containers in good condition as supplied by the manufacturer or formulator.   Pesticides shall be stored, handled, and used in accordance with manufacturer's labels.  Labels shall bear evidence of registration under the Federal Insecticide, Fungicide, and Rodenticide Act (FIFRA), as amended.

## 1.6	SAFETY REQUIREMENTS

The Contractor shall formulate, treat, and dispose of termiticides and their containers in accordance with label directions.  Water for formulating shall only come from sites designated by the Contracting Officer, and filling hose shall be fitted with a backflow preventer meeting local plumbing codes or standards.  Overflow shall be prevented during the filling operation.  Pesticides and related materials shall be kept under lock and key when unattended.  Proper protective clothing and equipment shall be worn and used during all phases of termiticide application.

## 1.7	WARRANTY

The Contractor shall provide a [3] [5] [_____]-year written warranty against infestations or reinfestations by subterranean termites of the buildings or building additions constructed under this contract.  Warranty shall include annual inspections of the building[s] or building addition[s].  If live subterranean termite infestation or subterranean termite damage is discovered during the warranty period, and the soil and building conditions have not been altered in the interim, the Contractor shall:

a.  Re-treat the soil and perform other treatment as may be necessary for elimination of subterranean termite infestation;

b.  Repair damage caused by termite infestation; and

c.  Reinspect the building approximately 180 days after the retreatment.

# 2	PRODUCTS

## 2.1	MATERIALS

Termiticides shall be currently registered by the EPA.

# 3	EXECUTION

## 3.1	VERIFICATION OF CONDITIONS

At the time of application, the soil moisture content shall be sufficiently low to allow uniform distribution of the treatment solution throughout the soil.  Applications shall not be made during or immediately following heavy rains or when conditions may cause runoff and create an environmental hazard.

## 3.2	APPLICATION

### 3.2.1	Treatment of New Structures

The Contractor shall establish complete and unbroken vertical and/or horizontal (as necessary) soil poison barriers between the soil and all portions of the intended structure which may allow termite access to wood and wood related products.  Termiticide shall be applied as a coarse spray and provide uniform distribution unto the soil surface.  Treatment shall be applied prior to placement of a vapor barrier or waterproof membrane and at least 12 hours prior to concrete placement.  Where treated soil or fill material is not to be covered with a vapor barrier or waterproof membrane, adequate precautions shall be taken to prevent its disturbance.  Soil or fill material disturbed after treatment shall be re-treated as specified above before placement of slabs or other covering structures.  Treatment of the soil on the exterior sides of foundation walls, grade beams, and similar structures shall be coordinated with final grading and planting operations so as to avoid disturbance of the treated barriers.   Manufacturer's warnings and precautions shall be observed in the handling and use of such materials.  Care shall be taken to prevent these chemicals from entering water supply systems, potable water supplies, or aquifers; and that they do not endanger plants or animals.  The Contracting Officer shall be notified at least 48 hours prior to beginning of treatment and formulating, mixing, and application shall be performed in the presence of the Contracting Officer's representative.

### 3.2.2	Rates and Methods of Application

Rates and methods of application shall be in accordance with the manufacturer's instructions on the pesticide label.  Maximum application or dosage rates shall be used.  If the pesticide contains less than the amount of active ingredient specified on the label, work shall be repeated with pesticides conforming to this specification.

## 3.3	DISPOSAL

The Contractor shall dispose of residual pesticides and containers off Government property in accordance with label instructions and EPA criteria.