SECTION 02082 - REMOVAL OF ASBESTOS-CONTAMINATED SOIL

THE PROCEDURES SET FORTH IN THIS SECTION ARE USEFUL IF A CONTAMINATED DIRT FLOOR IN A ROOM OR CRAWL SPACE IS ENCOUNTERED AS PART OF THE WORK. EDIT AS REQUIRED BY SPECIFIC PROJECT REQUIREMENTS.

PART 1 - GENERAL

1.1	RELATED DOCUMENTS:

A.	Drawings and general provisions of Contract, including General and Supplementary Conditions and other Division-1 Specification Sections, apply to work of this section.

1.2	RELATED WORK SPECIFIED ELSEWHERE:

A.	Asbestos abatement project requirements to be completed prior to start of the work of this section are set forth in the following sections:

1.	01503  Temporary Facilities - Asbestos Abatement

2.	01513  Temporary Pressure Differential & Air Circulation System

3.	01526  Temporary Enclosures - Complete Work Except Delete Floor Plastic.

4.	01560  Worker Protection - Asbestos abatement

5.	01562  Respiratory Protection

6.	01563  Decontamination Units

B.	Asbestos abatement project requirements to be completed at completion of the work of this section are set forth in the following sections:

1.	01711  Project Decontamination

SPECIFY THESE PRODUCTS HERE IF THE SECTION BELOW IS NOT USED.

C.	Amended water and removal encapsulant are specified in Section 02081 Removal of Asbestos-Containing Materials.

PART 2 - PRODUCTS (NOT APPLICABLE)

PART 3 - EXECUTION

3.1	PROCEDURES

A.	Wet Soils:  After removal of the Secondary Barrier and  floor layer of Primary Barrier, remove the top one inch (1")(25 mm) of soils which are damp or wet and place in disposal bags.  Start removal at the point of work farthest from the entrance to the soil floor area and proceed toward the entrance.  Do not permit traffic  into the fresh soil surface.  Arrange Pressure Differential System so that air flow is the starting point of work toward the entrance.  After the entire first layer of soil is removed completely change coveralls and at the entrance to the soil removal area don clean boot covers.  Remove the second one inch (1")(25 mm) of soil in the same manner as the first.  Carry out the decontamination procedures set forth in the "Project Decontamination" section of this specification at this time.  At the end of project decontamination remove the third one inch (1") (25 mm) of soil in the same manner as the previous two inches (2") (50 mm).

B.	Dry Soils:  Use the same procedure for dry soils, except saturate soil with amended water or a removal encapsulant as specified in other Division 2 sections of the specification.  If a removal encapsulant is used, use in accordance with manufacturer's instructions. Saturate soil beyond the inch of soil currently being removed.  If amended water is used keep the surface of the soil continuously wet throughout removal and decontamination.

DECISION ON WHETHER OR NOT TO ALLOW USE OF A REMOVAL ENCAPSULANT SHOULD BE MADE AFTER TESTING INVOLVED SOIL.

END OF SECTION  - 02082