SECTION 01421 - PROJECT CLEARANCE - LEAD-BASED PAINT

PART  - GENERAL

1.1	RELATED DOCUMENTS:

A.	Drawings and general provisions of  the Contract, including General and Supplementary Conditions and other Division - 1 Specification Sections, apply to work of this section.

1.	Visual Inspection:  Required as a prerequisite of sampling is set forth in Section 01715 Project Decontamination.

1.2	 DESCRIPTION OF THE WORK

A.	Not in Contract Sum:  This section describes work being performed by the Owner's Project Monitor.  This work is not in the Contract Sum.

B.	This section sets forth required surface lead dust concentration in the work area and describes testing procedures the Owner will use to measure these levels.

C.	Soil Testing:  This section sets forth required soil lead content measurements conducted on exterior abatement projects which will be used to:

1.	Support pre-and post-abatement comparisons, and

2.	Determine if statistically significant changes in soil lead content exist following completion of abatement.

1.3	ANALYTICAL METHODS:

A.	Atomic Absorption Spectroscopy or Inductively Coupled Plasma Emission Spectroscopy will be used for analysis of:

1.	Surface lead dust wipe samples

2.	Air Samples

3.	Soil Samples

1.4	VISUAL INSPECTION:

A.	Work of this section will not begin until the visual inspection described in section 01715 Project Decontamination has been completed and certified by the project monitor.

1.5	CLEARANCE CRITERIA:

A.	On-site Paint Removal:  Clearance will be conducted on three surfaces in each room or area.  One each from the floor, window sill and window trough.

B.	Off-site Paint Removal:  Substrate Removal, Enclosure Projects:.  Clearance will be conducted on one surface per room or area, equally divided among floors, window sills and window troughs.

C.	Wipe Sampling Clearance:  Decontamination is complete when every sample is at or below the following levels.  If clearance levels are not satisfactory, the decontamination is incomplete and recleaning per Section 01715 - Project Decontamination is required at no additional cost to the Owner.

1.	Floors:		100 micrograms per square foot.

2.	Window Sills:		500 micrograms per square foot.

3.	Window Troughs:	800 micrograms per square foot.

4.	Exterior:		800 micrograms per square foot

D.	Soil Sampling Clearance:  Remediation is complete when every sample is at or below the following levels.  If clearance levels are not satisfactory, the remediation is incomplete and additional remediation per section 02066 is required at no additional cost to the owner.

1. 	Soil:			400 parts per million

1.6.	SCHEDULE OF SAMPLES:  At the completion of hazard reduction the following samples will be collected.

| BUILDING | UNIT/ APARTMENT | FLOOR LEVEL LOCATION | ROOM/AREA DESIGNATION | SURFACE | NUMBER OF SAMPLES |
| --- | --- | --- | --- | --- | --- |
| SEVEN | 7-20 | GROUND | KITCHEN | FLOOR | ONE |
| SEVEN | 7-20 | GROUND | KITCHEN | WINDOW TROUGH | ONE |
| SEVEN | 7-20 | GROUND | KITCHEN | WINDOW TROUGH | ONE |
| EIGHT | 8-10 | SECOND | BATHROOM | FLOOR | ONE |
| EIGHT | 8-10 | SECOND | BEDROOM #1 | WINDOW TROUGH | ONE |
| EIGHT | 8-10 | SECOND | BEDROOM #2 | WINDOW SILL | ONE |
| EIGHT | 8-10 | EXTERIOR | PORCH | FLOOR | ONE |
| EIGHT | 8-10 | N/A | EXTERIOR OF BUILDING | SOIL | ONE COMPOSITE |

PART 2 - PRODUCT (NOT APPLICABLE)

PART 3 - EXECUTION (NOT APPLICABLE)

END OF SECTION - 01421